Distributed Edge Computing Plan

Plan C - YOLO + Jetson

2026-04-08

# Overview

Each funnel gets an independent Jetson + camera + light, performing capture, YOLO detection, and PLC write independently.

| Dimension | Current (Centralized) | Plan C (Distributed Edge) |
| --- | --- | --- |
| Compute | 1 IPC | 1 Jetson per funnel |
| Algorithm | Traditional CV | YOLO26 + CV fusion |
| Inference | ~51ms | ~8-15ms (TensorRT) |
| Fault isolation | One fails all stop | Independent |
| Robustness | Low (threshold) | High (deep learning) |

# Hardware per Node

| Component | Model | Price | Notes |
| --- | --- | --- | --- |
| Edge compute | Jetson Orin Nano 8GB | 2500 CNY | 40 TOPS, 15W |
| Camera | Basler acA1600-60gm | 4300 CNY | Existing, USB3/GigE |
| Light | LED industrial | 1000 CNY | PLC controlled |
| Enclosure | IP65 industrial | 500 CNY | Dust/water proof |
| Power | 12V/5A | 200 CNY | Camera + Jetson |
| Cable | Cat6 | 100 CNY | To switch |
| Total per node |  | 8600 CNY |  |

# Total Cost (1 tipper, 5 funnels)

| Item | Qty | Unit | Total |
| --- | --- | --- | --- |
| Edge nodes | 5 | 8600 | 43000 |
| PLC (1769-L16ER) | 1 | 15000 | 15000 |
| Switch | 1 | 500 | 500 |
| Grand total |  |  | 58500 CNY |

# PLC Tag Design

| Tag | Type | Description |
| --- | --- | --- |
| FunnelN_CanTip | BOOL | Funnel N can tip |
| FunnelN_FaultCode | DINT | Funnel N fault code |
| FunnelN_ResultValid | BOOL | Funnel N result valid |
| FunnelN_Heartbeat | DINT | Funnel N heartbeat (500ms) |
| FunnelN_Online | BOOL | Funnel N online |
| All_CanTip | BOOL | All funnels allow = tip |
| Any_Fault | BOOL | Any funnel has fault |
| Allow_Tip | BOOL | Final allow tip |

# YOLO Model Deployment

| Class | Meaning | Action |
| --- | --- | --- |
| grid_visible | Grid hole visible | Normal |
| coal_cover | Coal coverage | Alarm |
| shadow | Shadow (not coal) | Ignore |
| rust | Rust (not coal) | Ignore |
| foreign_object | Foreign object | Alarm |

# Dual Mode Fusion

| YOLO | CV | Fusion | Confidence |
| --- | --- | --- | --- |
| Coal | Coal | Coal | HIGH |
| Coal | No coal | Coal | MEDIUM |
| No coal | Coal | Manual confirm | LOW |
| No coal | No coal | No coal | HIGH |
| YOLO fail | Any | Use CV only | Degraded |

# Implementation Roadmap

| Phase | Time | Content | Deliverable |
| --- | --- | --- | --- |
| 1 | Week 1-2 | Buy 1 Jetson, port code | Single node verified |
| 2 | Week 3-4 | Collect 500+ images, train YOLO | coal_detect.engine |
| 3 | Week 5-6 | TensorRT deploy + fusion | Dual mode verified |
| 4 | Week 7-8 | PLC tag rework, funnel 1 trial | Single funnel online |
| 5 | Week 9-12 | Buy remaining 4, all online | Full system online |

# Risk Mitigation

| Risk | Impact | Mitigation |
| --- | --- | --- |
| Jetson overheating | Throttle | IP65 enclosure + heatsink |
| YOLO inaccurate | False alarms | Keep CV as fallback |
| Jetson failure | 1 funnel down | PLC heartbeat, CanTip=False |
| USB camera unstable | Capture loss | Keep GigE as backup |
| pycomm3 on ARM | PLC comm fail | Pure Python, should work |

# Cost Comparison

|  | Current | Plan C | Delta |
| --- | --- | --- | --- |
| Hardware | 50000 CNY | 58500 CNY | +17% |
| Inference | 51ms | 8-15ms | 3-6x faster |
| Accuracy | Medium | High | Major improvement |
| Fault isolation | All stop | Independent | Fundamental |
| Power | 65W | 75W | +10W |